## Standard Operating Procedure (SOP): Event Kiosk Rental IT Operations

Department: IT
 Version: 1.0
 Date: [7/2/2025]
 Scope: Applies to all IT staff responsible for preparing, deploying, and supporting kiosks for event rentals.

## 1. Purpose

To ensure all event kiosks are properly maintained, updated, securely packed, deployed, and supported, providing a seamless and secure user experience. This SOP covers every step of the IT department’s responsibilities in managing event kiosk rentals covering maintenance, content updates, deployment, on-site checks, client communication, and remote monitoring.

2. Responsibilities

- IT Specialist: Maintenance, software updates, hardware checks, packing, deployment, and onsite support.

- IT Interns: Assist IT Specialists with maintenance, updates, hardware checks, packing, deployment, and on-site support under supervision.

- Project Manager: Oversee the entire process, give final approvals, and act as the escalation contact for any issues. Coordinates between IT and the client for high-level issues or changes in scope. Assists with packing and delivering the Kiosk.

## 3. Procedure

## A. Pre-Event Preparation

- Maintenance & Upgrade

  - Check Kiosk availability based on the event/rental request.

  - Inspect hardware for physical damage or wear.

  - Clean and dust all components.

  - Update Ubuntu to the latest stable patches for 20.04.

  - Upgrade kiosk software and ensure all security patches are applied.(Note to add the updating procedure )

  - Check Google Chrome version is up to date with the current version.

  - Test peripherals (touchscreen, printers, Wi-Fi card, Sound)

- Content Update

  - Request for content: Request for final approved content to be uploaded to the kiosk

  - Upload New Content: Load the event-specific content (e.g., URL, videos, offline wordpress) onto the kiosk. This might involve downloading new files or deploying an updated content package provided by the client. (refer content update manual)

  - Functionality Test: Run the kiosk software with the new content and thoroughly test all media and interactive elements. Ensure videos play correctly, images and text display properly, and interactive features ( buttons, forms, links) function as intended.

- System Testing

  - Reboot and Monitor: Reboot the kiosk after updates and observe the startup sequence. Check that it boots directly into the kiosk application without errors. Look for any error messages or unusual behavior during startup/shutdown.

    - Manual method:

      - Open the back panel and relock it with Key #600.

      - Connect a USB Keyboard

      - Plug the keyboard into an available USB port inside the kiosk.

      - Open Terminal (Using Keyboard Shortcut) Press Ctrl + Alt + T (common Linux shortcut) to open the terminal. (If that doesn’t work, try Ctrl + Shift + T or Alt + F2 and type gnome-terminal.)

      - In the terminal, type:

      - sudo reboot

      - Enter the admin password if prompted (Bm3Admin).

      - Wait for the kiosk to restart.

      - After reboot, safely disconnect the keyboard.

      - Close the back panel and relock it with Key #600.

    - Through Meshcentral Terminal

      - Log in to Meshcentral through https://meshcentral.bm3group.com/

      - Open the kiosk by the corresponding kiosk name labeled at the back of each kiosk

      - Click on Terminal

      - Type reboot

  - Diagnostic Checks: Run any available diagnostics or health check tools. Verify that CPU, memory, and temperature are within normal ranges and that no critical errors appear in system logs.

    - Refer the Diagnostic check manual (kiosk_health.sh)

  - Network Connectivity: Connect the kiosk to an internal test network (or the event network if accessible in advance) to ensure it can get online. Confirm that all online features of the kiosk are working.

  - Remote Management Agent: Ensure the remote monitoring/management agent (e.g., MeshCentral or AnyDesk) is running and the kiosk is reporting online status to the management console. This will allow IT to monitor the kiosk’s health and perform remote support during the event. (If the agent is not checking in, troubleshoot network or agent configuration now.)

    - Through Meshcentral Terminal

      - Login to Meshcentral through https://meshcentral.bm3group.com/

      - Open the kiosk by correspondence kiosk name labled at the back of each kiosk

      - Click on Desktop and check if you can see the screen

      - You can run the kiosk_health.sh script at this point through termianl

  - Battery/Power (for Robot): verify the battery is charged and holding charge from the dashboard of the robot chust on the top right corner check for the battery symbole with persentage.

- Packing for Transport

Power Down: Shut down the kiosk properly via the operating system to prevent data corruption. Once fully off, disconnect all cables (power, Ethernet, peripheral connectors).

Protective Packing: Place the kiosk and its components into the designated transport cases. Use protective padding (foam, bubble wrap, etc.) around screens and sensitive parts to prevent damage in transit. Kiosks should be secured so they won’t shift during transport.

Accessories Checklist: Include all necessary accessories and spares:

- Power cables and adapters.

- Extension cords or power strips (if venue may need them).

- Ethernet cable (even if planning to use Wi-Fi, a wired backup might be useful).

- Any special adapters or mounts.

- Backup peripherals or spare parts (extra cables, a spare touchscreen stylus, spare printer WiFi Card, SSD,  etc.).

- A USB flash drive with backup of the software/content (in case re-installation is needed on site).

- Printed documentation: quick setup guide, troubleshooting tips, contact numbers for support.

Inventory Check: Use a checklist to inventory all items before sealing the case. This ensures nothing is forgotten. Have another team member cross-verify the packed items if possible.

## B. Event Deployment

- Unpacking & Setup

- Carefully unpack kiosk at event site.

- Assemble and position kiosk as per event requirements.

- Connect to power and network (Wi-Fi or Ethernet).

- Power on and perform a quick system check.

- Internet Connectivity

- Connect kiosk to the designated event network. The project manager will update you with the password.

- Test internet access and ensure remote management tools (e.g., MeshCentral agent) are running.

  - Through Meshcentral Terminal

    - Login to Meshcentral through https://meshcentral.bm3group.com/

    - Open the kiosk by correspondence kiosk name labled at the back of each kiosk

    - Click on Desktop and check if you can see the screen

- Final Onsite Check

  - Content Display Verification: Make sure the kiosk is presenting the correct content. Navigate through the screen flow as an end-user would. Check that all text, images, and videos appear as expected on the event site’s network. Sometimes content might behave differently on a new network or due to firewalls

    - Communicate with the project manager for any identified issue

  - – ensure everything loads.

  - conditions.

  - Performance Check: Observe the kiosk for a few minutes to confirm it’s running smoothly (no freezing or unexpected reboots). Check that the screen calibration is still accurate, volume levels are appropriate (if sound is used), and that the device isn’t overheating.

    - Troubleshoot using the diagnostic script

    - If problem persist use the backup computer

  - Client Walk-Through: If the client or event staff is present, do a quick walk-through of the kiosk with them. Show them it’s working and review any functionality they should know about. This is also a good time to inform them whom to contact (provide IT support contact info) if any issue arises during the event.

  - Issue Resolution: If any problems are discovered during setup (e.g., a portion of the content is not loading, or a device isn’t functioning), address them immediately. This could mean rebooting the kiosk, adjusting settings, or in a worst-case scenario, swapping out hardware. Always have a basic tool kit on hand (screwdrivers, cables, etc.) for quick fixes. If a kiosk unit is completely non-functional and you have a backup unit available, replace it on the spot to avoid downtime. It’s best practice to keep spare hardware components or an extra kiosk ready as a contingency

  - Ready State Confirmation: Once everything passes the final checks, confirm that the kiosk is event-ready. From this point, it should not require further adjustments— it is now in service for event attendees.

## C. During Event

- Monitor kiosk performance periodically.

  - Project manager or CEO will give direction if IT is required on site

  - Alwasys monitor the performance through Meshcentral

- Provide onsite or remote support as needed.

## D. Post-Event

- Shutdown & Packing

  - Power down kiosk safely.

  - Disconnect and pack all components.

  - Return kiosk to office.

- Return & Remote Monitoring

  - Reconnect kiosk in the office.

  - Use MeshCentral to perform post-event diagnostics and monitor for any issues.

  - Log all maintenance and incidents for record-keeping.

## 4. Troubleshooting & Escalation

- Refer to troubleshooting guide from basecamp IT folder for common issues.

- Escalate unresolved technical problems to Project Manager.

## 5. Documentation

- Complete maintenance and deployment checklists for each event.

- Record software versions, updates applied, and any incidents.

## 6. Review & Updates

- SOP is to be reviewed in two years or after major system changes.

Index:

- Checklists to ensure each step is completed and documented.

# Event Kiosk Rental IT Operations Checklist

Department: IT
 Version: 1.0
 Date: 7/2/2025
 Prepared by: [Name]
 Event Name/Location: __________________________
 Kiosk Serial/ID: __________________________

## A. Pre-Event Preparation

## Maintenance & Upgrade

- Inspect hardware for physical damage or wear

- Clean and dust all components

- Update Ubuntu to latest stable patches for 20.04

- Upgrade kiosk software and apply all security patches

- Check Google Chrome version and update if needed

- Test all peripherals:

  - Touchscreen

  - Printer(s)

  - Wi-Fi card

  - Sound

## Content Update

- Upload new event-specific content to kiosk

- Test all media and interactive elements for functionality

## System Testing

- Reboot kiosk and check for error messages

- Run hardware and software diagnostics

- Confirm network adapters and Wi-Fi modules are functional

## Packing for Transport

- Power down kiosk and disconnect all cables

- Securely pack kiosk in transport case with protective padding

- Include all necessary accessories:

  - Cables

  - Adapters

  - Backup USBs

  - Documentation (setup instructions, troubleshooting guide)

## B. Event Deployment

## Unpacking & Setup

- Carefully unpack kiosk at event site

- Document any damage ______________________________________

- Assemble and position kiosk as per event requirements

- Connect to power and network (Wi-Fi/Ethernet)

- Power on and perform a quick system check

## Internet Connectivity

- Connect kiosk to designated event network

- Test internet access

- Ensure MeshCentral agent (or other remote management tool) is running

## Final Onsite Check

- Confirm content displays correctly

- Test all user-facing features (touch, print, sound, etc.)

- Address any errors/issues before event start - record any adjustments made

## C. During Event

- Monitor kiosk performance periodically - Through meshcentral

- Provide onsite or remote support as needed

## D. Post-Event

## Shutdown & Packing

- Power down kiosk safely - define safely

- Disconnect and pack all components

- Return kiosk to office

## Return & Remote Monitoring

- Reconnect kiosk in the office

- Use MeshCentral to perform post-event diagnostics

- Log all maintenance and incidents for record-keeping

- If Wordpress offline used download analytics

## E. Troubleshooting & Escalation

- Refer to troubleshooting guide from Basecamp IT folder for common issues

- Escalate unresolved technical problems to Project Manager

## F. Documentation

- Complete this checklist and file in event records

- Record software versions, updates applied, and any incidents

Sign-off:
 IT Specialist: ____________________ Date: ___________
 IT Intern: _______________________ Date: ___________
 Project Manager: _________________ Date: ___________